[image: image4.jpg]


Element Minneapolis

	Data sheet
Highly Accelerated Life Test (HALT) 

	Company name
	Data Panel
	Performed by
	DJL/MJC
	Pages
	1 of 1

	Job number
	52911-8
	Specification 
	Customer

	DUT description
	DP Block and Mini Combo
	Test date(s)
	4/11/16 – 4/12/16



	Device under test information

	Description
	Model / part number
	Serial number

	DP Block and Mini Combo (2)
	N/A
	N/A

	
	
	


	Equipment list

	503-173
	204-542
	380-620
	
	
	

	400-070
	480-163
	
	
	
	


	Temperature
	Vibration
	Start
	Stop
	Comments / notes
	Date

	0°C
	0
	9:35
	9:50
	Test unit operation by customer
	4/11/16

	-10°C
	0
	9:55
	10:05
	
	4/11/16

	-20°C
	0
	10:09
	10:19
	Temperature controlled on UUT thermocouple
	4/11/16

	-30°C
	0
	10:20
	10:30
	
	4/11/16

	-40°C
	0
	10:31
	10:41
	
	4/11/16

	-50°C
	0
	10:41
	10:51
	
	4/11/16

	-60°C
	0
	10:52
	11:02
	
	4/11/16

	-70°C
	0
	11:03
	11:13
	
	4/11/16

	-80°C
	0
	11:14
	11:20
	UUT alarm went off, halted test
	4/11/16

	25°C
	0
	11:26
	
	Replaced test unit
	4/11/16

	30°C
	0
	11:42
	11:52
	
	4/11/16

	40°C
	0
	11:55
	12:05
	
	4/11/16

	50°C
	0
	12:08
	12:18
	Per customer, set temperature to 80°C
	4/11/16

	80°C
	0
	12:20
	12:30
	
	4/11/16

	90°C
	0
	12:32
	12:42
	Per customer. Change dwell time to 5 minutes
	4/11/16

	100°C
	0
	12:44
	12:49
	Per customer, return to ambient
	4/11/16

	25°C
	0
	1:05
	
	
	4/11/16

	-60°C
	0
	1:15
	1:25
	Thermal cycle #1
	4/11/16

	100°C
	0
	1:35
	1:45
	
	4/11/16

	-60°C
	0
	2:00
	2:10
	Thermal cycle #2
	4/11/16

	100°C
	0
	2:16
	2:26
	
	4/11/16

	-60°C
	0
	2:40
	2:50
	Thermal cycle #3
	4/11/16

	100°C
	0
	3:00
	3:10
	
	4/11/16

	25°C
	0
	
	
	Return to ambient
	4/11/16

	
	
	
	
	
	

	25°C
	10 grms
	10:17
	10:27
	Vibration at ambient (25°C)
	4/12/16

	25°C
	20 grms
	10:29
	10:39
	
	4/12/16

	25°C
	30 grms
	10:41
	10:51
	
	4/12/16

	25°C
	5 grms
	10:52
	10:52
	“tickle vibration”
	4/12/16

	25°C
	40 grms
	10:54
	11:04
	Vibration at ambient (continued)
	4/12/16

	25°C
	50 grms
	11:05
	11:15
	
	4/12/16

	25°C
	5 grms
	11:16
	11:16
	
	4/12/16

	25°C
	60 grms
	11:18
	11:28
	Could not reach 60 grms for full 10 minutes
	4/12/16

	25°C
	5 grms
	11:29
	11:29
	“tickle vibration”
	4/12/16

	25°C
	0
	11:29
	
	Replaced UUT for combined environment
	4/12/16

	-60°C
	10 grms
	11:46
	11:56
	
	4/12/16

	100°C
	10 grms
	12:01
	12:11
	
	4/12/16

	-60°C
	20 grms
	12:17
	12:27
	
	4/12/16

	100°C
	20 grms
	12:30
	12:40
	
	4/12/16

	25°C
	5 grms
	12:40
	12:41
	“tickle vibration” 
	4/12/16

	-60°C
	30 grms
	12:45
	
	UUT failure, return to ambient
	4/12/16

	25°C
	0
	
	
	
	4/12/16

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	


	Results

	


	Disposition
	|(   )| Retained at Element Minneapolis  
	|(   )| Returned to customer
	|(   )| Other (describe):  


Photograph 1:  DUT identification, XXXX

Photograph 2:  View of external equipment utilized during testing
Photograph 3:  View of test setup inside chamber
Photograph 4:  

Photograph 5:  

Photograph 6:  

Photograph 7:  

Photograph 8:  

Photograph 9:  

Photograph 10:  

Photograph 11:  

Photograph 12:  

[image: image1.emf]
Figure 1:  

[image: image2.emf]
Figure 2:  

[image: image3.emf]
Figure 3:  

	QAF 5.10 HALT Rev A DC5-DS87.dotx   |   11/10/15   |   
	


Element Minneapolis
EAR/ITAR-Controlled Data
This document is subject to the controls and restrictions on the cover page
Engineering Report No. XXXXXX

